Run Charts

Definitions/Purpose:  A run chart is a line graph that shows data points plotted in the order in which they occur.  They are used to focus on vital changes in a process by showing trends and shifts in a process over time, variation over time, or to identify/monitor decline or improvement in a process over time.  Typically used during the Measure phase.

Instructions:

- Decide on process improvement measure.

- Gather data, generally 2-25 data pint to detect meaningful patterns.

- Create graph with vertical line (Y axis) and Horizontal axis (X axis)

  - Draw scale related to variable you are measuring on Y axis, arranging to cover 1/12 times the range of your data

  - Draw time or sequence scale on horizontal line (x-axis)

  - Can easily be done through excel functions

- Plot the data and calculate mean (sum of data points divided by # of data points).  If data are asymmetric – calculate median.

- Interpret the chart (use caution in interpretation of < 20 data points)

  - Note position of mean or median- is it relative to customer need or specification?  Is it where you want it relative to your objectives?

  - Count runs above and below mean or median (series of points on same side of mean or median.

    - 6-14 runs are normal for 20 data points

      - variations from this indicate special causes in the process

      - too few runs – indicates  shift in process average, cycle, or trend

      - too many runs – indicate sampling from 2 sources or bias

      - when counting runs- omit any points that repeat preceding value and nay points on the centerline

  - 7 or > point in a row continuously increasing or decreasing (trend)

    - when counting – omit any points on centerline. It does not make or break the run, nor does it add to the length of the run.

  - 14 or > points in a row alternating up and down

    - can be counted anywhere on chart; doesn’t need to be on same side of mean or median

    - omit any points that repeat preceding value. Such points do not add to length of the run or do they break it.

## Example of Run Chart